Configuring Multi-Area OSPF Network

Eric Xia

Purpose

The purpose of this lab is to configure OSPF between neighbouring networks. Additionally, this also adds in multi-area OSPFv2 and OSPFv3 for long-distance simulation, and multi-layer switches for more complex routing scenarios.

Background

OSPF, which stands for Open Shortest Path First, is a widely used interior gateway routing protocol that has played a critical role in the evolution of IP networking. It was initially developed in the late 1980s by the Internet Engineering Task Force (IETF) as a replacement for the aging Routing Information Protocol (RIP). At the time, RIP had been a simple and reliable protocol for basic network routing, but it was increasingly unable to handle the demands of growing and more complex network topologies. RIP’s limitations, including its relatively slow convergence time, maximum hop count, and lack of support for advanced routing features, made it inadequate for modern networks. As a result, the development of a more sophisticated routing protocol became necessary.

OSPF addressed these limitations by providing a more advanced method for discovering network paths and calculating the best routes for packet delivery. Unlike RIP, which is a distance-vector protocol that relies on hop counts, OSPF is a link-state protocol. This means that each OSPF-enabled router maintains a complete map of the network, known as a link-state database. To determine the shortest and most efficient paths through the network, OSPF uses Dijkstra’s algorithm, which calculates the least-cost path from a router to all other routers in the network. This approach allows OSPF to respond more quickly to network changes and to scale efficiently across large and complex network environments.

Another major advancement in OSPF is its support for load balancing through Equal-Cost Multipath (ECMP) routing. With ECMP, multiple paths of equal cost to a destination can be used simultaneously, improving bandwidth utilization and providing redundancy in case of link failures. Additionally, OSPF included built-in authentication mechanisms that ensured routers exchanged routing information securely, a significant improvement over RIP, which had no native security features. This made OSPF both faster and more secure, offering a better foundation for enterprise networks that required reliability and data integrity.

The original version of OSPF, known as OSPFv2, was specifically designed for IPv4 routing. OSPFv2 also introduced the concept of areas, which allowed network administrators to divide an OSPF network into hierarchical segments. This supported both single-area and multi-area network designs, reducing the size of the routing table and limiting the scope of network updates to improve efficiency. OSPFv2 also used Link-State Advertisements (LSAs) to communicate the state of each router and its connected links. LSAs contained critical information such as IP addresses and network topology, allowing each router to build an accurate and up-to-date map of the network.

With the eventual development and deployment of IPv6, OSPF needed to evolve to accommodate the new protocol. This led to the introduction of OSPFv3 in 1998, which was designed to handle IPv6 routing while retaining many of the features that made OSPFv2 successful. OSPFv3 includes several notable differences from its predecessor. First, while OSPFv3 is primarily an IPv6 protocol, it can also route IPv4 traffic through extensions, making it versatile in mixed network environments. Second, instead of using OSPF’s internal authentication methods, OSPFv3 relies on IPsec to provide robust security, offering stronger protection for routing information. Third, OSPFv3 allows multiple OSPF processes to run on the same network interface, which provides additional flexibility in network design. Finally, it uses an IPv6 header rather than an IPv4 header for packet encapsulation, reflecting the protocol’s native support for IPv6.

Overall, the development of OSPF represented a major leap forward in the field of IP routing. Its combination of link-state routing, efficient path calculation, support for hierarchical network structures, load balancing, and security features made it a significant improvement over RIP. Even decades after its introduction, OSPF remains a foundational routing protocol, widely implemented and actively maintained across both enterprise and service provider networks. Its continued relevance demonstrates the robustness and forward-thinking design of the protocol, which successfully adapted to the transition from IPv4 to IPv6 while maintaining high levels of efficiency, scalability, and security.

Lab Summary

In this lab, my lab partner and I set up multi-area OSPFv2 and OSPFv3, configuring 5 routers and 2 multi-layer switches (Layer 3 switches), for a total of 7 devices. Area 0 is the backbone, with 3 devices in total, with one central device, and 2 devices that border upon 2 other areas (Areas 1 and 2). In Areas 1 and 2, there are the 2 border devices that are linked between Area 0 and Area 1 or 2, and then linked to another router, which is then linked to a multi-layer switch on each end. Each of the devices are configured with one or two IPv4 and an IPv6 address depending on how many ports are linked to the networks. Then we configured with the same router ID for both OSPFv2 and OSPFv3 on process ID 1, along with linking network statements into the interfaces that have the IPv4 and IPv6 addresses. We also put in basic security commands, such as a MOTD banner and a line con 0 password. Afterwards, we test out whether the commands work by pinging to confirm the connection is established and then traceroute to confirm that the connection is going through each OSPF area thru the backbone.

Lab Commands

Standard commands (OSPF commands only):

Router3(config)#router ospf 1

Router3(config-router)#router-id 3.3.3.3

Router3(config)#ipv6 router ospf 1

Router3(config-rtr)#router-id 3.3.3.3

Router3(config)#ipv6 unicast-routing

Router3(config-if)#ip address 192.168.2.2 255.255.255.252

Router3(config-if)#ipv6 address 2001:db8:67:abcd::2/36

Switch(config)# sdm prefer dual-ipv4-and-ipv6 default

Router3(config)#router ospf 1

Router3(config-router)#router-id 3.3.3.3

Router3(config-if)#ip ospf 1 area 0

Router3(config-if)#ipv6 ospf 1 area 0

For the multi-layer switches:

switch1(config-if)#no switchport

Network Diagrams

IPv4 Network Topology:

IPv6 Network Topology:

Step-by-step Guide Perchance

Step 0

There are a few things that you would want to mind. Perchance. This includes the proper wiring connections into the right ports on the 5 routers and 2 switches.

Boot into the router. When, prompted to enter the configuration dialog, type in No, n, or no. It will always bypass the dialogue. Perchance.

Just in case, reload the router again if there is a configuration on it already, and you want to get rid of it:

Router>enable

Router#reload

Proceed with reload? [confirm]

*Sep 11 09:03:11.169: %SYS-5-RELOAD: Reload requested by console. Reload Reason: Reload Command.

*NOTE: you could just power cycle the router by using the power button. That is, if you have access to it, perchance.

After reloading the router, you’re prompted with the configuration dialog, which you do again. This time, you’ll know that the configs are empty, and you’re free to start configuring the routers.

Step 1

Now, go back into enable, and now, enter configure mode by using the configure terminal command:
Router>en

Router#conf t

Now, set a hostname for the router.

Router(config)#hos [insert hostname]

[insert hostname](config)#

*NOTE: in this case, we’ll be using rx as the hostname, so that’ll be what you see for the commands. Also, it’s normal for me to shorten the commands by a ton, so you’ll see that here. Perchance.

rx(config)#

Now, it’s time to configure basic security on the router. Let’s first start off with a banner motd command, and then set a line con 0 password along with encrypting it with service password-encryption:
rx(config)#banner motd &pls don’t enter (°-°)&

rx(config)#lin co 0

rx(config-line)#pass cisco

rx(config-line)#exi

rx(config)#ser password-e

Now, you’ve set the router’s basic security and passwords. Perchance.

Step 2

Time to configure basic IP configurations! Perchance.

First, get into each of the interfaces that you’re going to use and are connected to each other. On the routers, it’s g0/0/0 and g0/0/1. Also, remember to set ipv6 unicast-routing for the ipv6 configurations to go thru properly.

rx(config)#ipv u

rx(config)#int g0/0/0

rx(config-if)#ip add 169.67.167.69 255.255.255.252

*NOTE: use the IP addresses that you use and chose for that specific port. Subnetting will always be .252, or /30. Same goes for IPv6, where the subnetting will always be /126, but the address will be different.

rx(config-if)#ipv add 2001:69:67:420::1/126

rx(config-if)#no sh

Repeat the same on g0/0/1. Perchance.

Step 3

Time to work on the OSPF configuration.

Exit out of the interface, the go into router ospf. Then, set the router id. Do the same in ipv6 router ospf.

rx(config-int)#exi

rx(config)#router ospf 1

rx(config-router)#router- [x.x.x.x]

rx(config-router)#ipv router os 1

rx(config-rtr)#router- [x.x.x.x]

*NOTE: [x.x.x.x] is different for each router. For example, router 4 has an id of 4.4.4.4, while router 2 has an id of 2.2.2.2.

Now, it’s time to put the network statements into the interfaces. I use [a] and [b] for the OSPF areas. If the router is only in a single area, then [a] and [b] are the same. Make sure to refresh the router ospf process afterwards and save the config into a file that is NOT running-config, unless if the router is your own and you need OSPF on the router by default. Perchance.

rx(config-rtr)#int g0/0/0

rx(config-if)#ip os 1 a [a]

rx(config-if)#ipv os 1 a [a]

rx(config-if)#int g0/0/1

rx(config-if)#ip os 1 a [b]

rx(config-if)#ipv os 1 a [b]

rx(config-if)#end

rx#cle ip os pro

rx#copy ru flash:[insert filename here]

OR

rx#copy ru st

(if you can do so and won’t get in trouble doing so)

Congratulations! You’ve set a single router up. Now, make sure to repeat these another 4 times on each different router. Now, the configs are mostly the same, besides the hostname, ip addresses and ipv6 addresses, and OSPF areas. Perchance.

Step 4

Time to configure the 2 switches in the network. Yipee. Perchance.

Please first do reset the switches (refer to Step 0, since the process is the same).

Here are the commands (just in case you’re too lazy to go back to Step 0):

Switch>enable

Switch#reload

Proceed with reload? [confirm]

*Sep 11 09:03:11.169: %SYS-5-RELOAD: Reload requested by console. Reload Reason: Reload Command.

Step 5

Alright, now you’ve finished that, it’s time to configure the basic switch security configs.

It’s the same as before, but here they are, perchance:

Switch>en

Switch#conf t

Now, set a hostname for the router.

Switch(config)#hos sd
sd(config)#banner motd &pls don’t enter (°-°)&

sd(config)#lin co 0

sd(config-line)#pass cisco

sd(config-line)#exi

sd(config)#ser password-e

Step 6

Time to configure basic IP configurations… again. Perchance.

On the switches, somehow, it’s g1/0/1. This is probably because I’m using a multi-layer switch, also known as a layer 3 switch. This allows for ipv6 address configuration and most router configs. If you do end up using standard single-layer switches (layer 2 switches), then the port will be f0/1. Again, I just used example addresses here. One more thing here, it’s that you’re going to use the ports and configure ip and ipv6 addresses onto them, then the no switchport command is needed. Perchance.

sd(config)#ipv u

sd(config)#int g1/0/1

sd(config-if)#no swi

sd(config-if)#ip add 169.69.167.14 255.255.255.252

sd(config-if)#ipv add 2001:169:167:420::1/126

sd(config-if)#no sh

Step 7

It’s OSPF config time. The process is mostly the same, but with less stuff.

sd(config-int)#exi

sd(config)#router ospf 1

sd(config-router)#router- [x.x.x.x]

sd(config-router)#ipv router os 1

sd(config-rtr)#router- [x.x.x.x]

*NOTE: Yes, switches, out of all things, will need router-IDs. Surprised? Perchance.

sd(config-rtr)#int g1/0/1

sd(config-if)#ip os 1 a [n]

sd(config-if)#ipv os 1 a [n]

sd(config-if)#end

sd#cle ip os pro

sd#copy ru flash:[insert filename here]

OR

sd#copy ru st

(if you can do so and won’t get in trouble doing so)

It’s time to do testing to see if you really did set the stuff properly.

Step 8

Testing time!

On routers, you only really need to test out if the OSPF areas are working or not.

Using traceroute on switch, you will get something like this:

Tracing the route to 192.168.3.6

VRF info: (vrf in name/id, vrf out name/id)

1 192.168.1.2 4 msec 4 msec 0 msec

2 192.168.1.6 0 msec 4 msec 0 msec

3 192.168.2.2 0 msec 4 msec 0 msec

4 192.168.2.6 0 msec 4 msec 0 msec

5 192.168.3.2 4 msec 0 msec 4 msec

6 192.168.3.6 112 msec 4 msec *

If you first traceroute, then you don’t need to ping afterwards.

Then, you go onto the routers to check if the areas are working and valid or not.

OSPF Router with ID (6.6.6.6) (Process ID 1)

Router Link States (Area 1)

Link ID         ADV Router      Age         Seq#       Checksum Link count

1.1.1.1         1.1.1.1         755         0x80000005 0x007EE0 2

2.2.2.2         2.2.2.2         101         0x80000004 0x00A797 1

6.6.6.6         6.6.6.6         745         0x80000003 0x001118 1

Net Link States (Area 1)

Link ID         ADV Router      Age         Seq#       Checksum

192.168.1.2     1.1.1.1         756         0x80000001 0x00EDC2

192.168.1.5     1.1.1.1         816         0x80000001 0x0007B6

Summary Net Link States (Area 1)

Link ID         ADV Router      Age         Seq#       Checksum

192.168.2.0     2.2.2.2         101         0x80000001 0x007D4E

192.168.2.4     2.2.2.2         57          0x80000001 0x005F67

192.168.3.0     2.2.2.2         51          0x80000001 0x008642

192.168.3.4     2.2.2.2         51          0x80000001 0x00685B

You also should check ip routes on both switches and routers. It should look something like this:

192.168.1.0/30 is subnetted, 2 subnets

O IA 192.168.1.0 [110/6] via 192.168.3.5, 00:21:27, GigabitEthernet1/0/1

O IA 192.168.1.4 [110/5] via 192.168.3.5, 00:21:27, GigabitEthernet1/0/1

192.168.2.0/30 is subnetted, 2 subnets

O IA 192.168.2.0 [110/4] via 192.168.3.5, 00:21:27, GigabitEthernet1/0/1

O IA 192.168.2.4 [110/3] via 192.168.3.5, 00:22:07, GigabitEthernet1/0/1

192.168.3.0/24 is variably subnetted, 3 subnets, 2 masks

O 192.168.3.0/30 [110/2] via 192.168.3.5, 00:23:30, GigabitEthernet1/0/1

C 192.168.3.4/30 is directly connected, GigabitEthernet1/0/1

L 192.168.3.6/32 is directly connected, GigabitEthernet1/0/1

Ipv6 routes look similar, but instead of having an O for OSPF, it has an Oi for OSPFv3 instead. Here are what the pings and traceroutes look like for ipv6:

Router2#ping 2001:db8:11::2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:11:ABCD::2, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/5/16 ms

s2#traceroute 2001:db8:11::2

Type escape sequence to abort.

Tracing the route to 2001:DB8:11:ABCD::2

1 2001:DB8:169:ABCD::1 0 msec 0 msec 0 msec

2 2001:DB8:420:ABCD::2 4 msec 0 msec 0 msec

3 2001:DB8:167:ABCD::2 4 msec 0 msec 0 msec

4 2001:DB8:67:ABCD::1 4 msec 0 msec 4 msec

5 2001:DB8:69:ABCD::1 0 msec 4 msec 0 msec

6 2001:DB8:11:ABCD::2 24 msec 4 msec 0 msec

s2#

Congratulations! You’ve completed the setup for OSPFv2 and OSPFv3 for an entire backbone network, and verified that it works.

Lab Configurations

R1:

Current configuration : 1661 bytes

! Last configuration change at 18:15:28 UTC Fri Sep 12 2025

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname Router1

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

ipv6 unicast-routing

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FLM2408005M

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface GigabitEthernet0/0/0

ip address 192.168.1.2 255.255.255.252

ip ospf 1 area 1

negotiation auto

ipv6 address 2001:DB8:11:ABCD::1/126

ipv6 ospf 1 area 1

interface GigabitEthernet0/0/1

ip address 192.168.1.5 255.255.255.252

ip ospf 1 area 1

negotiation auto

ipv6 address 2001:DB8:69:ABCD::1/126

ipv6 ospf 1 area 1

interface GigabitEthernet0/2/0

no ip address

negotiation auto

interface GigabitEthernet0/2/1

no ip address

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

negotiation auto

router ospf 1

router-id 1.1.1.1

ip forward-protocol nd

no ip http server

ip http secure-server

ip tftp source-interface GigabitEthernet0

ipv6 router ospf 1

router-id 1.1.1.1

control-plane

banner motd ^Cdo not enter^C

line con 0

password cisco

login

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

End

R2:

Current configuration : 1661 bytes

! Last configuration change at 17:42:35 UTC Fri Sep 12 2025

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname Router2

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

ipv6 unicast-routing

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FLM24060912

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface GigabitEthernet0/0/0

ip address 192.168.1.6 255.255.255.252

ip ospf 1 area 1

negotiation auto

ipv6 address 2001:DB8:69:ABCD::2/126

ipv6 ospf 1 area 1

interface GigabitEthernet0/0/1

ip address 192.168.2.1 255.255.255.252

ip ospf 1 area 0

negotiation auto

ipv6 address 2001:DB8:67:ABCD::1/126

ipv6 ospf 1 area 0

interface GigabitEthernet0/2/0

no ip address

negotiation auto

interface GigabitEthernet0/2/1

no ip address

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

negotiation auto

router ospf 1

router-id 2.2.2.2

ip forward-protocol nd

no ip http server

ip http secure-server

ip tftp source-interface GigabitEthernet0

ipv6 router ospf 1

router-id 2.2.2.2

control-plane

banner motd ^Cdo not enter^C

line con 0

password cisco

login

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

R3:

Current configuration : 1618 bytes

! Last configuration change at 18:33:50 UTC Thu Sep 11 2025

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

no platform punt-keepalive disable-kernel-core

hostname Router3

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

ipv6 unicast-routing

subscriber templating

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO214420G7

spanning-tree extend system-id

redundancy

mode none

vlan internal allocation policy ascending

interface GigabitEthernet0/0/0

ip address 192.168.2.2 255.255.255.252

ip ospf 1 area 0

negotiation auto

ipv6 address 2001:DB8:67:ABCD::2/126

ipv6 ospf 1 area 0

interface GigabitEthernet0/0/1

ip address 192.168.2.5 255.255.255.252

ip ospf 1 area 0

negotiation auto

ipv6 address 2001:DB8:167:ABCD::2/126

ipv6 ospf 1 area 0

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

interface Vlan1

no ip address

shutdown

router ospf 1

router-id 3.3.3.3

ip forward-protocol nd

no ip http server

no ip http secure-server

ip tftp source-interface GigabitEthernet0

ipv6 router ospf 1

router-id 3.3.3.3

control-plane

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

End

R4

Current configuration : 1609 bytes

! Last configuration change at 17:27:02 UTC Fri Sep 12 2025

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

no platform punt-keepalive disable-kernel-core

hostname R4

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

ipv6 unicast-routing

subscriber templating

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO211216BL

spanning-tree extend system-id

redundancy

mode none

vlan internal allocation policy ascending

interface GigabitEthernet0/0/0

ip address 192.168.3.1 255.255.255.252

ip ospf 1 area 0

negotiation auto

ipv6 address 2001:DB8:420:ABCD::2/126

ipv6 ospf 1 area 0

interface GigabitEthernet0/0/1

ip address 192.168.2.6 255.255.255.252

negotiation auto

ipv6 address 2001:DB8:167:ABCD::1/126

ipv6 ospf 1 area 2

interface Serial0/1/0

no ip address

interface Serial0/1/1

no ip address

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

negotiation auto

interface Vlan1

no ip address

router ospf 1

router-id 4.4.4.4

ip forward-protocol nd

no ip http server

no ip http secure-server

ip tftp source-interface GigabitEthernet0

ipv6 router ospf 1

router-id 4.4.4.4

control-plane

line con 0

password 7 1511021F0725

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

R5

Current configuration : 1516 bytes

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

no platform punt-keepalive disable-kernel-core

hostname Router

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-famil

no aaa new-model

ipv6 unicast-routing

subscriber templating

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO214421CF

spanning-tree extend system-id

redundancy

mode none

vlan internal allocation policy ascendin

interface GigabitEthernet0/0/0

ip address 192.168.3.2 255.255.255.252

ip ospf 1 area 2

negotiation auto

ipv6 address 2001:DB8:420:ABCD::1/126

ipv6 ospf 1 area 2

interface GigabitEthernet0/0/1ip address 192.168.3.5 255.255.255.252

ip ospf 1 area 2

negotiation auto

ipv6 address 2001:DB8:169:ABCD::1/126

ipv6 ospf 1 area 2

interface Serial0/1/0

no ip address

interface Serial0/1/

no ip address

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip addres

negotiation auto

interface Vlan1

no ip address

router ospf 1

router-id 5.5.5.5

ip forward-protocol nd

no ip http server

no ip http secure-server

ip tftp source-interface GigabitEthernet0

ipv6 router ospf 1

router-id 5.5.5.5

control-plane

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

End

MLS1

Current configuration : 9213 bytes

! Last configuration change at 17:38:44 UTC Fri Sep 12 2025

version 16.12

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

! Call-home is enabled by Smart-Licensing.

service call-home

platform punt-keepalive disable-kernel-core

hostname switch1

vrf definition Mgmt-vrf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

switch 1 provision c9200l-24t-4g

ip routing

login on-success log

ipv6 unicast-routing

call-home

! If contact email address in call-home is configured as sch-smart-licensing@cisco.com

! the email address configured in Cisco Smart License Portal will be used as contact email address to send SCH notifications.

contact-email-addr sch-smart-licensing@cisco.com

profile "CiscoTAC-1"

active

destination transport-method http

no destination transport-method email

no device-tracking logging theft

diagnostic bootup level minimal

spanning-tree mode rapid-pvst

spanning-tree extend system-id

memory free low-watermark processor 10055

redundancy

mode sso

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

shutdown

negotiation auto

interface GigabitEthernet1/0/1

no switchport

ip address 192.168.1.1 255.255.255.252

ip ospf 1 area 1

ipv6 address 2001:DB8:11:ABCD::2/126

ipv6 ospf 1 area 1

interface GigabitEthernet1/0/2

interface GigabitEthernet1/0/3

interface GigabitEthernet1/0/4

interface GigabitEthernet1/0/5

interface GigabitEthernet1/0/6

interface GigabitEthernet1/0/7

interface GigabitEthernet1/0/8

interface GigabitEthernet1/0/9

interface GigabitEthernet1/0/10

interface GigabitEthernet1/0/11

interface GigabitEthernet1/0/12

interface GigabitEthernet1/0/13

interface GigabitEthernet1/0/14

interface GigabitEthernet1/0/15

interface GigabitEthernet1/0/16

interface GigabitEthernet1/0/17

interface GigabitEthernet1/0/18

interface GigabitEthernet1/0/19

interface GigabitEthernet1/0/20

interface GigabitEthernet1/0/21

interface GigabitEthernet1/0/22

interface GigabitEthernet1/0/23

interface GigabitEthernet1/0/24

interface GigabitEthernet1/1/1

interface GigabitEthernet1/1/2

interface GigabitEthernet1/1/3

interface GigabitEthernet1/1/4

interface Vlan1

no ip address

router ospf 1

router-id 6.6.6.6

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ipv6 router ospf 1

router-id 6.6.6.6

control-plane

service-policy input system-cpp-policy

line con 0

password cisco

login

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

line vty 5 15

login

end

MLS2

Current configuration : 8657 bytes

! Last configuration change at 18:06:46 UTC Fri Sep 12 2025

version 16.9

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

! Call-home is enabled by Smart-Licensing.

service call-home

no platform punt-keepalive disable-kernel-core

hostname s2

vrf definition Mgmt-vrf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

switch 1 provision c9200l-24t-4g

ip routing

ip arp entry learn 10240

login on-success log

ipv6 unicast-routing

call-home

! If contact email address in call-home is configured as sch-smart-licensing@cisco.com

! the email address configured in Cisco Smart License Portal will be used as contact email address to send SCH notifications.

contact-email-addr sch-smart-licensing@cisco.com

profile "CiscoTAC-1"

active

destination transport-method http

no destination transport-method email

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

crypto pki trustpoint TP-self-signed-2221393201

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-2221393201

revocation-check none

rsakeypair TP-self-signed-2221393201

license boot level network-essentials addon dna-essentials

diagnostic bootup level minimal

spanning-tree mode rapid-pvst

spanning-tree extend system-id

redundancy

mode sso

policy-map system-cpp-policy

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

shutdown

negotiation auto

interface GigabitEthernet1/0/1

no switchport

ip address 192.168.3.6 255.255.255.252

ip ospf 1 area 2

ipv6 address 2001:DB8:169:ABCD::2/126

ipv6 ospf 1 area 2

interface GigabitEthernet1/0/2

interface GigabitEthernet1/0/3

interface GigabitEthernet1/0/4

interface GigabitEthernet1/0/5

interface GigabitEthernet1/0/6

interface GigabitEthernet1/0/7

interface GigabitEthernet1/0/8

interface GigabitEthernet1/0/9

interface GigabitEthernet1/0/10

interface GigabitEthernet1/0/11

interface GigabitEthernet1/0/12

interface GigabitEthernet1/0/13

interface GigabitEthernet1/0/14

interface GigabitEthernet1/0/15

interface GigabitEthernet1/0/16

interface GigabitEthernet1/0/17

interface GigabitEthernet1/0/18

interface GigabitEthernet1/0/19

interface GigabitEthernet1/0/20

interface GigabitEthernet1/0/21

interface GigabitEthernet1/0/22

interface GigabitEthernet1/0/23

interface GigabitEthernet1/0/24

interface GigabitEthernet1/1/1

interface GigabitEthernet1/1/2

interface GigabitEthernet1/1/3

interface GigabitEthernet1/1/4

interface Vlan1

no ip address

shutdown

router ospf 1

router-id 7.7.7.7

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ipv6 router ospf 1

router-id 7.7.7.7

control-plane

service-policy input system-cpp-policy

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

line vty 5 15

login

end

Testing Results

Traceroute from MLS1 to MLS2

OSPFv2

Tracing the route to 192.168.3.6

VRF info: (vrf in name/id, vrf out name/id)

1 192.168.1.2 4 msec 4 msec 0 msec

2 192.168.1.6 0 msec 4 msec 0 msec

3 192.168.2.2 0 msec 4 msec 0 msec

4 192.168.2.6 0 msec 4 msec 0 msec

5 192.168.3.2 4 msec 0 msec 4 msec

6 192.168.3.6 112 msec 4 msec *

OSPFv3 (MLS2 to MLS1)

Type escape sequence to abort.

Tracing the route to 2001:DB8:11:ABCD::2

1 2001:DB8:169:ABCD::1 0 msec 0 msec 0 msec

2 2001:DB8:420:ABCD::2 4 msec 0 msec 0 msec

3 2001:DB8:167:ABCD::2 4 msec 0 msec 0 msec

4 2001:DB8:67:ABCD::1 4 msec 0 msec 4 msec

5 2001:DB8:69:ABCD::1 0 msec 4 msec 0 msec

6 2001:DB8:11:ABCD::2 24 msec 4 msec 0 msec

Multi-area OSPF area database

OSPFv2

OSPF Router with ID (6.6.6.6) (Process ID 1)

Router Link States (Area 1)

Link ID         ADV Router      Age         Seq#       Checksum Link count

1.1.1.1         1.1.1.1         755         0x80000005 0x007EE0 2

2.2.2.2         2.2.2.2         101         0x80000004 0x00A797 1

6.6.6.6         6.6.6.6         745         0x80000003 0x001118 1

Net Link States (Area 1)

Link ID         ADV Router      Age         Seq#       Checksum

192.168.1.2     1.1.1.1         756         0x80000001 0x00EDC2

192.168.1.5     1.1.1.1         816         0x80000001 0x0007B6

Summary Net Link States (Area 1)

Link ID         ADV Router      Age         Seq#       Checksum

192.168.2.0     2.2.2.2         101         0x80000001 0x007D4E

192.168.2.4     2.2.2.2         57          0x80000001 0x005F67

192.168.3.0     2.2.2.2         51          0x80000001 0x008642

192.168.3.4     2.2.2.2         51          0x80000001 0x00685B

OSPFv3

OSPFv3 Router with ID (2.2.2.2) (Process ID 1)

Router Link States (Area 0)

ADV Router       Age         Seq#        Fragment ID  Link count  Bits

2.2.2.2         152         0x80000006  0            1           B

3.3.3.3         157         0x80000008  0            2           None

4.4.4.4         158         0x80000009  0            1           B

Net Link States (Area 0)

ADV Router       Age         Seq#        Link ID    Rtr count

3.3.3.3         162         0x80000001  6          2

4.4.4.4         165         0x80000001  6          2

Inter Area Prefix Link States (Area 0)

ADV Router       Age         Seq#        Prefix

2.2.2.2         156         0x80000001  2001:DB8:69:ABCD::/126

2.2.2.2         156         0x80000002  2001:DB8:11:ABCD::/126

4.4.4.4         205         0x80000001  2001:DB8:169:ABCD::/126

4.4.4.4         205         0x80000001  2001:DB8:420:ABCD::/126

Link (Type-8) Link States (Area 0)

ADV Router       Age         Seq#        Link ID    Interface

2.2.2.2         152         0x80000002  7          Gi0/0/1

3.3.3.3         209         0x80000002  6          Gi0/0/1

Intra Area Prefix Link States (Area 0)

ADV Router       Age         Seq#        Link ID    Ref-lstype  Ref-LSID

3.3.3.3         162         0x80000001  6144       0x2002      6

4.4.4.4         165         0x80000001  6144       0x2002      6

Router Link States (Area 1)

ADV Router       Age         Seq#        Fragment ID  Link count  Bits

1.1.1.1         157         0x8000000E  0            2           None

2.2.2.2         154         0x80000007  0            1           B

6.6.6.6         271         0x80000007  0            1           None

192.168.2.1     499         0x80000003  0            1           B

Net Link States (Area 1)

ADV Router       Age         Seq#        Link ID    Rtr count

1.1.1.1         157         0x80000003  7          2

6.6.6.6         275         0x80000003  38         2

Inter Area Prefix Link States (Area 1)

ADV Router       Age         Seq#        Prefix

2.2.2.2         156         0x80000002  2001:DB8:167:ABCD::/126

2.2.2.2         156         0x80000002  2001:DB8:67:ABCD::/126

2.2.2.2         156         0x80000002  2001:DB8:420:ABCD::/126

2.2.2.2         156         0x80000002  2001:DB8:169:ABCD::/126

192.168.2.1     504         0x80000001  2001:DB8:67:ABCD::/126

Link (Type-8) Link States (Area 1)

ADV Router       Age         Seq#        Link ID    Interface

1.1.1.1         316         0x80000001  7          Gi0/0/0

2.2.2.2         152         0x80000002  6          Gi0/0/0

Intra Area Prefix Link States (Area 1)

ADV Router       Age         Seq#        Link ID    Ref-lstype  Ref-LSID

1.1.1.1         157         0x80000003  7168       0x2002      7

6.6.6.6         275         0x80000003  38912      0x2002      38

IP routes (only got IPv4)

192.168.1.0/30 is subnetted, 2 subnets

O IA     192.168.1.0 [110/6] via 192.168.3.5, 00:21:27, GigabitEthernet1/0/1

O IA     192.168.1.4 [110/5] via 192.168.3.5, 00:21:27, GigabitEthernet1/0/1

192.168.2.0/30 is subnetted, 2 subnets

O IA     192.168.2.0 [110/4] via 192.168.3.5, 00:21:27, GigabitEthernet1/0/1

O IA     192.168.2.4 [110/3] via 192.168.3.5, 00:22:07, GigabitEthernet1/0/1

192.168.3.0/24 is variably subnetted, 3 subnets, 2 masks

O        192.168.3.0/30

[110/2] via 192.168.3.5, 00:23:30, GigabitEthernet1/0/1

C        192.168.3.4/30 is directly connected, GigabitEthernet1/0/1

L        192.168.3.6/32 is directly connected, GigabitEthernet1/0/1

Issues

We had quite a few issues here. While we got OSPFv2 to run properly on first try, we couldn’t get OSPFv3 to run properly and was trying to figure out what the issue was on. Before that, though, me and my lab partner figured out that I got the 2 interfaces on r4 mixed up, which resulted in OSPF routing errors. Afterwards, when we were trying to get OSPFv3 to work, it resulted in the traceroutes and pings all timing out. When we checked the single-area traceroutes in Area 1 and Area 2, they worked, but when we checked the configs for Area 0 in the r2 configurations, there was an error in the configurations, where the router-ids were nowhere to be found, and r1 (centre of the layout) was missing it entirely (if I remember right). This resulted in the Area 0 layout to be broken up into 2 parts, and I had to reconfigure Area 0’s issues. We then also figured out that r2 was also misconfigured–the area configs were mixed up on the interfaces–which resulted in us fixing that too. After that, everything was fixed, and the traceroutes worked (it took a while to get things up and running, since portfast was off…).

Another miscellaneous issue that we had was the router version mismatch. 3 of our routers (r3, r4, and r5) were version 15.5, while the other 2 routers were version 16.9. We couldn’t fix this issue by updating the router versions (oof), so we found 3 other routers that were indeed version 16.9 (what basically is r6, r7, r8) and moved all the configs there. This fixed the issue where the OSPF routing was messed up, even though the configs were entirely correct. Please note, though, we did get rid of all the OSPF network statements in the router ospf 1 interface as a just-in-case, since it wasn’t needed, and probably interfered with the in-physical-interface commands. Our final issue was that every time we loaded up the configs onto the routers, the interfaces were shut down, meaning we had to turn them on every single time. This wasn’t really an issue–it was more of an inconvenience–but it is a nice thing to note.

Conclusion

Completing this lab gave me confidence on how to run OSPFv2 and OSPFv3 in multi-area configurations, along with helping me improve on how to continue writing these labs. This also helped me figure out my troubleshooting skills, which gave me better understanding on how the show commands work. It also gave me better perception, since before that, a ton of the troubleshooting was done with the help of the mentors, but there no longer is the help of them, which gave me more experience. Overall, though, this was a very fun process of configuring multi-area OSPF, and it is not that hard to do it again.