Setting Up BGP Between Networks

By Eric Xia

Purpose

The purpose of this lab is to configure BGP to work with three different networks, specifically OSPF, EIGRP, and IS-IS (very poorly named, I know). Additionally, this will feature both IPv4 and IPv6 configurations for each, and the end goal is to ping from one end of the topology to the other using the BGP configurations.

Background

BGP

BGP, also known as Border Gateway Protocol, is an external gateway routing protocol that is designed to connect different routing protocols together and create routes in between routers that don’t have other routing protocols between them. BGP essentially works as a bridge between 2 bordering protocols and as a translator between the 2 protocols, providing routes between the borders. By doing so, you can ping a router that is 5 routers and 3 routing protocols away from you, along with connecting each branch of a company’s network together, especially if they run on different routing protocols.

BGP essentially was developed starting in the late-1980s, back when there still were very simple routing protocols (such as EGP). In fact, in 1989, BGP-1 was introduced to replace the aging, old, and far too simple EGP. During its early years, BGP evolved very quickly, with BGP-2 releasing in 1990 and BGP-3 releasing in 1991 to fix issues and improve on previous iterations. In 1994, BGP-4 releases, and adds support for Classless Inter-Domain Routing (CIDR) along with introducing the modern path-vector model. In the late 90s, BGP-4 gets improved on, but stays, with variations such as MP-BGP (for IPv6) and improvements such as traffic engineering, communities, route filtering, and policy control. Nowadays, BGP is known to have some security issues, but improvements on the security side for BGP come out and are known as RPKI and BGPsec.

OSPF

OSPF, also known as Open Shortest Path First, is an interior gateway routing protocol that is one of the most widely used protocols out there, known for minimising routing distance between routers. It is known for using the SPF/Dijkstra compute model for routing. Oftentimes, it would the shortest and most direct route using its routing table, which then goes to the second shortest route if the shortest route goes down, and so on and so on until it reaches its gateway of last resort. The reason why it is so popular is because it has an extremely high scalability, with both tiny 10-person companies and huge corporates using it. It can also be separated into single-area OSPF and multi-area OSPF, with multi-area OSPF being better for larger networks, whereas single-area OSPF would be better for closer integration.

OSPF is also known for having fast convergence, measuring using bandwidth (measured in cost), being classless, being hierarchical (with area 0 being the backbone), and supporting ECMP (load-balancing).

EIGRP

EIGRP, also known as Enhanced Interior Gateway Routing Protocol, is an interior gateway routing protocol that is mainly used inside Cisco-heavy organisations. It initially was Cisco-proprietary but eventually opened and now can be used on all different brands of routers and is currently partially open-source. It’s often called a hybrid protocol, since it uses both distance-vector routing and link state behaviour.

It works by neighbours sending partial updates, picking best paths and then good backup paths, and maintaining 3 tables: a neighbour table, a topology table, and a routing table. Like OSPF, it uses bandwidth to calculate its metrics, but also uses delay, which makes it faster than OSPF when it comes to route switching. It’s has its advantages over OSPF, with it being way faster when it comes to failover latency and has a very simple configuration in Cisco-heavy networks. It isn’t as popular compared to OSPF, because it’s historically known to be limited depending on the brand, making it much less common in multi-branded router environments (with not many Cisco routers or EIGRP-compatible routers), and it also has a lack of areas, which OSPF does not lack.

IS-IS

IS-IS, also known as Intermediate System to Intermediate System, is a catastrophically named (I mean who came up with this abbreviation) interior gateway routing protocol whose name as aged as well as Windows Millenium Edition right after Windows XP released, often used inside autonomous systems. It is very popular with ISPs and large service provider networks due to its ability to run in autonomous systems. It is also very well known that its name has poorly aged due to its association with a very certain organisation.

That aside, it works by treating routers as intermediary systems that exchange link-state PDUs, and that everyone builds from the same topology database. Something else to note is that updates in IS-IS is event-driven and not periodic. Its design is like OSPF in which it uses a two-level hierarchy, where Level-1 is intra-area routing, and Level-2 is inter-area routing. It is also classless and supports CIDR, just like OSPF. It also supports ECMP, and only use cost for its metrics, making it extremely predictable. It is used often due to its stability at scale, less updates than OSPF, and runs directly on layer 2, meaning that both IPv4 and IPv6 can run natively. Suspicious name aside, it does have its downsides since it looks weird compared to OSPF, and that it is much harder to configure compared to EIGRP or OSPF. Also, despite being one of the most stable and most convenient IGPs in existence, IS-IS still suffers from a very catastrophic naming abbreviation that creates a lot of discomfort and at least one “no, it’s not that organisation” clarification per discussion.

Lab Summary

In this lab, my lab partner and I set up 3 2-router networks that are each connected to each other, totalling up to 6 devices. The first 2 routers ran on OSPF, the next 2 routers ran on EIGRP, and the final 2 routers ran on IS-IS. There was a BGP link between routers 2 and 3, and another BGP link between routers 3 and 4. We also ran OSPFv3 on the first 2 routers and EIGRP for IPv6 on the next 2 routers. Since IS-IS ran IPv6 natively, we didn’t need to configure any extra protocol for it. That aside, we also did the basic router configs for each router, which included naming the router, setting security passwords for each router, and setting up MOTD for them. Afterwards, we tested to see if the pings went through from the first router to the last router and then verified that it was using BGP by tracerouting it and showing the IP routes.

Network Diagrams

IPv4:

IPv6:

Step-by-step guide

Step 1

First, we start off with the most basic router configurations. This includes setting passwords and naming the switch (please first do enable and then conf t!)

This applies to every single router

| Command | Description |
| --- | --- |
| hostname xxx | Sets the hostname of the device |
| line con 0 password cisco login exit | Sets the device password |
| line vty 0 15 password cisco login exit | Sets the line vty 0 4 and line vty 5 15 passwords (both at the same time) |
| enable secret class | Sets the password for entering privileged exec mode |
| banner motd #unauthorised access is prohibited and can be punishable# | Sets the banner message that shows when entering the device |
| ipv6 unicast-routing | Allows for the IPv6 addresses to be set inside interfaces |

Step 2

Now it’s time to set the rest of the basic configs onto each of the routers. This includes the IPv4 and IPv6 addresses, and on the proper ports they should be on.

IPv4:

|  | R1 | R2 | R3 | R4 | R5 | R6 |
| --- | --- | --- | --- | --- | --- | --- |
| G0/0/0 | 192.168.1.1 | 182.168.1.2 | 192.168.1.6 | 192.168.2.2 | 192.168.3.2 | 192.168.3.6 |
| G0/0/1 | N/A | 192.168.1.5 | 192.168.2.1 | 192.168.3.1 | 192.168.3.5 | N/A |
| Lo0 | N/A | N/A | 3.3.3.3 | 4.4.4.4 | 5.5.5.5 | 6.6.6.6 |

Note that for all g0/0/x  subnets are 255.255.255.252 (/30), and for all loopback interfaces subnets are 255.255.255.255 (/32)

IPv6:

|  | R1 | R2 | R3 | R4 | R5 | R6 |
| --- | --- | --- | --- | --- | --- | --- |
| G0/0/0 | 2001:db8:11:abcd::2 | 2001:db8:11:abcd::1 | 2001:db8:69:abcd::2 | 2001:db8:67:abcd::2 | 2001:db8:420:abcd::1 | 2001:db8:169:abcd::2 |
| G0/0/1 | N/A | 2001:db8:69:abcd::1 | 2001:db8:67:abcd::1 | 2001:db8:420:abcd::2 | 2001:db8:169:abcd::1 | N/A |

Note that for all g0/0/x  subnets are /126.

Commands:

| Command | Description |
| --- | --- |
| int g0/0/x ip add 192.168.x.x 255.255.255.252 ipv add 2001:db8:xxx:abcd::x/126 no shut | Sets the IPv4 and IPv6 address for each interface. |
| int lo0 ip add x.x.x.x 255.255.255.255 | Sets the loopback interface (primarily for troubleshooting) |

Step 3

Now, it’s time to set each of the individual internal gateway protocols. Let’s start with OSPF on routers 1 and 2.

| Router | Router-ID |
| --- | --- |
| R1 | 1.1.1.1 |
| R2 | 2.2.2.2 |

Here are the commands:

| Command | Description |
| --- | --- |
| router ospf 1 router-id x.x.x.x redistribute connected | Sets the router ID for OSPF and redistributes all the IP routes |
| ipv6 router ospf 1 router-id x.x.x.x redistribute connected | Sets the router ID for OSPFv3 and redistributes all the IPv6 routes |
| int g0/0/0 ip ospf 1 area 0 ipv6 ospf 1 area 0 | Connects OSPF and OSPFv3 into the desired interface |

Step 4

Now, it’s time to set EIGRP on routers 3 and 4.

| Command | Description |
| --- | --- |
| router eigrp 100 network 3.3.3.3 0.0.0.0 network 4.4.4.4 0.0.0.0 network 192.168.2.0 0.0.0.3 passive-interface default no passive-interface g0/0/x no passive-interface lo0 | Sets the EIGRP networks, also disables passive interface for certain interfaces so it shows on the routes. It’s g0/0/1 for R3 and g0/0/0 for R4. |
| int g0/0/x bandwidth 100000 ip eigrp 100 ipv6 eigrp 100 | Sets the interface up for EIGRP. It’s g0/0/1 for R3 and g0/0/0 for R4. |

Now, note here that the EIGRP uses loopback interfaces for troubleshooting.

Step 5

Now, it’s time to set up IS-IS on routers 5 and 6.

| Command | Description |
| --- | --- |
| router isis 1 net xx.000x.0000.0000.000x.00 is-type level-1 metric-style wide redistribute connected | Sets up the IS-IS setup, also redistributing the routes at the same time Sets up a net entry title for the router Manually set IS-IS to run on level-1 and not level-1-2 |
| int g0/0/x ip router isis 1 | Adds in IS-IS into the specified interface. It’s g0/0/1 for R5 and g0/0/0 for R6. |
| int lo0 ip router isis 1 | Adds in IS-IS into the loopback interface for troubleshooting reasons. |

Step 6

Now, it’s time to set up BGP between routers.

Let’s work with configurations between R2 and R3 and between R4 and R5.

| Command | Description |
| --- | --- |
| router bgp 6500x bgp log-neighbor-changes neighbor 2001:DB8:xx:ABCD::x remote-as 6500x neighbor 192.168.x.x remote-as 6500x | Sets up BGP for the router (not set in interface yet), sets the neighbour address for both IPv4 and IPv6. The BGP number is 65001 for R2, 65002 for R3, 65003 for R4, and 65004 for R5. The neighbouring router will be the number that isn’t assigned to the router itself and instead will be the other number. |
| (in router bgp 6500x) address-family ipv4 redistribute connected redistribute ospf 1 neighbor 192.168.x.x activate | Sets the neighbours and address family for IPv4, along with redistributing the routes from the OSPF side of things. |
| address-family ipv6 redistribute connected redistribute ospf 1 neighbor 2001:DB8:xx:ABCD::x activate | Sets the neighbours and address family for IPv4, along with redistributing the routes from the OSPFv3 side of things. |

Note that all interfaces here apply for every one of them.

Step 7

Since each router runs a different IGP, the configs will be slightly different.

R2

| Command | Description |
| --- | --- |
| router ospf 1 redistribute connected subnets redistribute bgp 65001 subnets | Redistributes the discovered BGP routes into the routing database for IPv4, allowing for OSPF to route outside of the network. |
| ipv6 router ospf 1 redistribute connected redistribute bgp 65001 | Redistributes the discovered BGP routes into the routing database for IPv6, allowing for OSPFv3 to route outside of the network. |

R3 and R4

| Command | Description |
| --- | --- |
| router eigrp 100 redistribute bgp 6500x metric 10000 100 255 1 1500 redistribute connected | Redistributes the BGP routes into the routing database for EIGRP to work with, allowing for EIGRP to route outside the network for both IPv4 and IPv6. The BGP number is 65002 for R3 and 65003 for R4. |

R5

| Command | Description |
| --- | --- |
| router isis 1 redistribute connected redistribute bgp 65003 metric 1 level-1 | Redistributes the BGP routes into the routing database for IS-IS to work with, allowing for IS-IS to route outside of the network for both IPv4 and IPv6. |

Congratulations! You have completed setup for BGP. Now, you only really need to do certain commands to see if the configurations work. Use traceroute and show ip route to verify that it’s working.

Configs

R1 (OSPF):

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R1

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success lo

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing multilink bundle-name

spanning-tree extend system-id

Redundancy

mode non

interface GigabitEthernet0/0/0

ip address 192.168.1.1 255.255.255.252

ip ospf 1 area 0

negotiation auto

ipv6 address 2001:DB8:11:ABCD::2/126

ipv6 ospf 1 area 0

interface GigabitEthernet0/0/1

no ip address

negotiation auto

interface GigabitEthernet0/2/0

no ip address

negotiation auto

interface GigabitEthernet0/2/1

no ip address

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

negotiation auto

router ospf 1

router-id 1.1.1.1

ip forward-protocol nd

no ip http server

ip http secure-server

ipv6 router ospf 1

router-id 1.1.1.1

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

R2 (OSPF + BGP):

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R2

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FLM24060912

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface GigabitEthernet0/0/0

ip address 192.168.1.2 255.255.255.252

ip ospf 1 area 0

negotiation auto

ipv6 address 2001:DB8:11:ABCD::1/126

Ipv6 ospf 1 area 0

interface GigabitEthernet0/0/1

ip address 192.168.1.5 255.255.255.252

negotiation auto

ipv6 address 2001:DB8:69:ABCD::1/126

interface GigabitEthernet0/2/0

no ip address

negotiation auto

interface GigabitEthernet0/2/1

no ip address

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

negotiation auto

router ospf 1

router-id 2.2.2.2

redistribute connected subnets

redistribute bgp 65001 subnets

router bgp 65001

bgp log-neighbor-changes

neighbor 2001:DB8:69:ABCD::2 remote-as 65002

neighbor 192.168.1.6 remote-as 65002

address-family ipv4

redistribute connected

redistribute ospf 1

no neighbor 2001:DB8:69:ABCD::2 activate

neighbor 192.168.1.6 activate

exit-address-family

address-family ipv6

redistribute connected

redistribute ospf 1

neighbor 2001:DB8:69:ABCD::2 activate

exit-address-family

ip forward-protocol nd

no ip http server

ip http secure-server

ip tftp source-interface GigabitEthernet0

ipv6 router ospf 1

router-id 2.2.2.2

redistribute connected

redistribute bgp 65001

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

R3 (BGP + EIGRP):

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname r3

boot-start-marker

boot system bootflash:isr4300-universalk9.16.09.08.SPA.bin

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

Address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO214420G7

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

description eigrp test

ip address 3.3.3.3 255.255.255.255

interface GigabitEthernet0/0/0

description bgp

ip address 192.168.1.6 255.255.255.252

negotiation auto

ipv6 address 2001:DB8:69:ABCD::2/126

interface GigabitEthernet0/0/1

description eigrp

Bandwidth 100000

ip address 192.168.2.1 255.255.255.252

negotiation auto

ipv6 address 2001:DB8:67:ABCD::1/126

ipv6 eigrp 100

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router eigrp 100

network 3.3.3.3 0.0.0.0

network 4.4.4.4 0.0.0.0

network 192.168.2.0 0.0.0.3

redistribute bgp 65002 metric 10000 100 255 1    1500

redistribute connected

passive-interface default

no passive-interface GigabitEthernet0/0/1

no passive-interface Loopback0

router bgp 65002

bgp log-neighbor-changes

neighbor 2001:DB8:69:ABCD::1 remote-as 65001

neighbor 192.168.1.5 remote-as 65001

neighbor 192.168.2.2 remote-as 65002

address-family ipv4

redistribute connected

redistribute eigrp 100

no neighbor 2001:DB8:69:ABCD::1 activate

neighbor 192.168.1.5 activate

neighbor 192.168.1.5 weight 500

neighbor 192.168.2.2 activate

neighbor 192.168.2.2 weight 300

exit-address-family

address-family ipv6

redistribute connected

redistribute eigrp 100

neighbor 2001:DB8:69:ABCD::1 activate

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

ip route 192.168.2.2 255.255.255.255 192.168.2.2 90

ipv6 router eigrp 100

redistribute connected

redistribute bgp 65002 metric 10000 100 255 1 1500

route-map BGP-TO-EIGRP permit 10

set metric 1000000 10 255 1 1500

set tag 52

route-map EIGRP-TO-BGP deny 5

match tag 52

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

R4 (EIGRP + BGP):

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname r4

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO211216BL

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

description eigrp test

ip address 4.4.4.4 255.255.255.255

interface GigabitEthernet0/0/0

description eigrp

bandwidth 100000

ip address 192.168.2.2 255.255.255.252

negotiation auto

ipv6 address 2001:DB8:67:ABCD::2/126

ipv6 eigrp 100

interface GigabitEthernet0/0/1

description bgp

ip address 192.168.3.1 255.255.255.252

negotiation auto

ipv6 address 2001:DB8:420:ABCD::2/126

interface Serial0/1/0

no ip address

interface Serial0/1/1

no ip address

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

negotiation auto

router eigrp 100

network 3.3.3.3 0.0.0.0

network 4.4.4.4 0.0.0.0

network 192.168.2.0 0.0.0.3

redistribute bgp 65002 metric 10000 100 255 1  1500

no passive-interface GigabitEthernet0/0/0

no passive-interface Loopback0

router bgp 65002

bgp log-neighbor-changes

neighbor 2001:DB8:420:ABCD::1 remote-as 65003

neighbor 192.168.2.1 remote-as 65002

neighbor 192.168.3.2 remote-as 65003

address-family ipv4

redistribute eigrp 100

no neighbor 2001:DB8:420:ABCD::1 activate

neighbor 192.168.2.1 activate

neighbor 192.168.2.1 weight 300

neighbor 192.168.3.2 activate

neighbor 192.168.3.2 weight 500

neighbor 192.168.3.2 route-map PREF=ISIS in

exit-address-family

address-family ipv6

redistribute eigrp 100

neighbor 2001:DB8:420:ABCD::1 activate

exit-address-family

ip forward-protocol nd

no ip http server

ip http secure-server

ipv6 router eigrp 100

redistribute connected

redistribute bgp 65002 metric 10000 100 255 1 1500

route-map PREF=ISIS permit 10 set local-preference 300

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

R5 (BGP + IS-IS):

Version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R5

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO214421CF

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 5.5.5.5 255.255.255.255

ip router isis 1

interface GigabitEthernet0/0/0

ip address 192.168.3.2 255.255.255.252

negotiation auto

ipv6 address 2001:DB8:420:ABCD::1/126

interface GigabitEthernet0/0/1

ip address 192.168.3.5 255.255.255.252

ip router isis 1

negotiation auto

ipv6 address 2001:DB8:169:ABCD::1/126

interface Serial0/1/0

no ip address

interface Serial0/1/1

no ip address

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

negotiation auto

router isis 1

net 49.0001.0000.0000.0005.00

is-type level-1

metric-style wide

redistribute connected

redistribute bgp 65003 metric 1 level-1

address-family ipv6

redistribute connected

redistribute bgp 65003

exit-address-family

router bgp 65003

bgp log-neighbor-changes

neighbor 2001:DB8:420:ABCD::2 remote-as 65002

neighbor 192.168.3.1 remote-as 65002

address-family ipv4

redistribute connected

redistribute isis 1 level-1

no neighbor 2001:DB8:420:ABCD::2 activate

neighbor 192.168.3.1 activate

exit-address-family

address-family ipv6

redistribute connected

redistribute isis 1 level-1

network 2001:DB8:67:ABCD::/126

neighbor 2001:DB8:420:ABCD::2 activate

exit-address-family

ip forward-protocol nd

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

R6 (IS-IS):

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R6

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO21441WBL

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 6.6.6.6 255.255.255.255

ip router isis 1

interface GigabitEthernet0/0/0

ip address 192.168.3.6 255.255.255.252

ip router isis 1

shutdown

negotiation auto

ipv6 address 2001:DB8:168:ABCD::2/126

interface GigabitEthernet0/0/1

no ip address

shutdown

negotiation auto

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

interface GigabitEthernet0/2/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/2/1

no ip address

shutdown

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router isis 1

net 49.0001.0000.0000.0006.00

is-type level-1

metric-style wide

router isis

metric-style narrow

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

Test Results

Traceroute

R1 to R6

R1#traceroute 192.168.3.6

Tracing the route to 192.168.3.6

VRF info: (vrf in name/id, vrf out name/id)

1 192.168.1.2 1 msec 1 msec 0 msec

2 192.168.1.6 1 msec 1 msec 1 msec

3 192.168.2.2 1 msec 1 msec 1 msec

4 192.168.3.2 1 msec 2 msec 1 msec

5 192.168.3.6 1 msec 2 msec *

R6 to R1

R6#traceroute 192.168.1.1

Tracing the route to 192.168.1.1

VRF info: (vrf in name/id, vrf out name/id)

1 192.168.3.5 2 msec 1 msec 0 msec

2 192.168.3.1 0 msec 1 msec 1 msec

3 192.168.2.1 1 msec 1 msec 1 msec

4 192.168.1.5 2 msec 1 msec 1 msec

5 192.168.1.1 2 msec 2 msec *

Routing Tables

R1:

3.0.0.0/32 is subnetted, 1 subnets

O E2     3.3.3.3 [110/1] via 192.168.1.2, 23:10:38, GigabitEthernet0/0/0

4.0.0.0/32 is subnetted, 1 subnets

O E2     4.4.4.4 [110/1] via 192.168.1.2, 23:10:09, GigabitEthernet0/0/0

5.0.0.0/32 is subnetted, 1 subnets

O E2     5.5.5.5 [110/1] via 192.168.1.2, 23:04:38, GigabitEthernet0/0/0

6.0.0.0/32 is subnetted, 1 subnets

O E2     6.6.6.6 [110/1] via 192.168.1.2, 00:07:47, GigabitEthernet0/0/0

192.168.1.0/24 is variably subnetted, 3 subnets, 2 masks

C        192.168.1.0/30 is directly connected, GigabitEthernet0/0/0

L        192.168.1.1/32 is directly connected, GigabitEthernet0/0/0

O E2     192.168.1.4/30

[110/20] via 192.168.1.2, 23:10:50, GigabitEthernet0/0/0

192.168.2.0/30 is subnetted, 1 subnets

O E2     192.168.2.0 [110/1] via 192.168.1.2, 23:10:38, GigabitEthernet0/0/0

192.168.3.0/30 is subnetted, 1 subnets

O E2     192.168.3.4 [110/1] via 192.168.1.2, 00:08:17, GigabitEthernet0/0/0

R2:

3.0.0.0/32 is subnetted, 1 subnets

B        3.3.3.3 [20/0] via 192.168.1.6, 23:17:57

4.0.0.0/32 is subnetted, 1 subnets

B        4.4.4.4 [20/153856] via 192.168.1.6, 23:17:27

5.0.0.0/32 is subnetted, 1 subnets

B        5.5.5.5 [20/281856] via 192.168.1.6, 23:11:56

6.0.0.0/32 is subnetted, 1 subnets

B        6.6.6.6 [20/281856] via 192.168.1.6, 00:15:05

192.168.1.0/24 is variably subnetted, 4 subnets, 2 masks

C        192.168.1.0/30 is directly connected, GigabitEthernet0/0/0

L        192.168.1.2/32 is directly connected, GigabitEthernet0/0/0

C        192.168.1.4/30 is directly connected, GigabitEthernet0/0/1

L        192.168.1.5/32 is directly connected, GigabitEthernet0/0/1

192.168.2.0/30 is subnetted, 1 subnets

B        192.168.2.0 [20/0] via 192.168.1.6, 23:17:57

192.168.3.0/30 is subnetted, 1 subnets

B        192.168.3.4 [20/281856] via 192.168.1.6, 00:15:35

R3:

3.0.0.0/32 is subnetted, 1 subnets

C        3.3.3.3 is directly connected, Loopback0

4.0.0.0/32 is subnetted, 1 subnets

D        4.4.4.4 [90/153856] via 192.168.2.2, 23:19:28, GigabitEthernet0/0/1

5.0.0.0/32 is subnetted, 1 subnets

D EX     5.5.5.5 [170/281856] via 192.168.2.2, 23:13:39, GigabitEthernet0/0/1

6.0.0.0/32 is subnetted, 1 subnets

D EX     6.6.6.6 [170/281856] via 192.168.2.2, 00:16:47, GigabitEthernet0/0/1

192.168.1.0/24 is variably subnetted, 3 subnets, 2 masks

B        192.168.1.0/30 [20/0] via 192.168.1.5, 23:19:40

C        192.168.1.4/30 is directly connected, GigabitEthernet0/0/0

L        192.168.1.6/32 is directly connected, GigabitEthernet0/0/0

192.168.2.0/24 is variably subnetted, 2 subnets, 2 masks

C        192.168.2.0/30 is directly connected, GigabitEthernet0/0/1

L        192.168.2.1/32 is directly connected, GigabitEthernet0/0/1

192.168.3.0/30 is subnetted, 1 subnets

D EX     192.168.3.4

[170/281856] via 192.168.2.2, 00:17:17, GigabitEthernet0/0/1

R4:

3.0.0.0/32 is subnetted, 1 subnets

D        3.3.3.3 [90/153856] via 192.168.2.1, 23:19:55, GigabitEthernet0/0/0

4.0.0.0/32 is subnetted, 1 subnets

C        4.4.4.4 is directly connected, Loopback0

5.0.0.0/32 is subnetted, 1 subnets

B        5.5.5.5 [20/0] via 192.168.3.2, 23:14:10

6.0.0.0/32 is subnetted, 1 subnets

B        6.6.6.6 [20/20] via 192.168.3.2, 00:17:17

192.168.1.0/30 is subnetted, 2 subnets

D EX     192.168.1.0

[170/281856] via 192.168.2.1, 23:19:55, GigabitEthernet0/0/0

D EX     192.168.1.4

[170/26112] via 192.168.2.1, 23:19:55, GigabitEthernet0/0/0

192.168.2.0/24 is variably subnetted, 2 subnets, 2 masks

C        192.168.2.0/30 is directly connected, GigabitEthernet0/0/0

L        192.168.2.2/32 is directly connected, GigabitEthernet0/0/0

192.168.3.0/24 is variably subnetted, 3 subnets, 2 masks

C        192.168.3.0/30 is directly connected, GigabitEthernet0/0/1

L        192.168.3.1/32 is directly connected, GigabitEthernet0/0/1

B        192.168.3.4/30 [20/0] via 192.168.3.2, 00:17:47

R5:

3.0.0.0/32 is subnetted, 1 subnets

B        3.3.3.3 [20/153856] via 192.168.3.1, 23:17:27

4.0.0.0/32 is subnetted, 1 subnets

B        4.4.4.4 [20/0] via 192.168.3.1, 23:17:27

5.0.0.0/32 is subnetted, 1 subnets

C        5.5.5.5 is directly connected, Loopback0

6.0.0.0/32 is subnetted, 1 subnets

i L1     6.6.6.6 [115/20] via 192.168.3.6, 00:21:05, GigabitEthernet0/0/1

192.168.1.0/30 is subnetted, 2 subnets

B        192.168.1.0 [20/281856] via 192.168.3.1, 23:17:27

B        192.168.1.4 [20/26112] via 192.168.3.1, 23:17:27

192.168.2.0/30 is subnetted, 1 subnets

B        192.168.2.0 [20/0] via 192.168.3.1, 23:17:27

192.168.3.0/24 is variably subnetted, 4 subnets, 2 masks

C        192.168.3.0/30 is directly connected, GigabitEthernet0/0/0

L        192.168.3.2/32 is directly connected, GigabitEthernet0/0/0

C        192.168.3.4/30 is directly connected, GigabitEthernet0/0/1

L        192.168.3.5/32 is directly connected, GigabitEthernet0/0/1

R6:

3.0.0.0/32 is subnetted, 1 subnets

i L1     3.3.3.3 [115/11] via 192.168.3.5, 00:22:00, GigabitEthernet0/0/0

4.0.0.0/32 is subnetted, 1 subnets

i L1     4.4.4.4 [115/11] via 192.168.3.5, 00:22:00, GigabitEthernet0/0/0

5.0.0.0/32 is subnetted, 1 subnets

i L1     5.5.5.5 [115/20] via 192.168.3.5, 00:22:00, GigabitEthernet0/0/0

6.0.0.0/32 is subnetted, 1 subnets

C        6.6.6.6 is directly connected, Loopback0

192.168.1.0/30 is subnetted, 2 subnets

i L1     192.168.1.0 [115/11] via 192.168.3.5, 00:22:00, GigabitEthernet0/0/0

i L1     192.168.1.4 [115/11] via 192.168.3.5, 00:22:00, GigabitEthernet0/0/0

192.168.2.0/30 is subnetted, 1 subnets

i L1     192.168.2.0 [115/11] via 192.168.3.5, 00:22:00, GigabitEthernet0/0/0

192.168.3.0/24 is variably subnetted, 2 subnets, 2 masks

C        192.168.3.4/30 is directly connected, GigabitEthernet0/0/0

L        192.168.3.6/32 is directly connected, GigabitEthernet0/0/0

BGP Table for R4

r4#show bgp topology

BGP table version is 12, local router ID is 4.4.4.4

Network          Next Hop            Metric LocPrf Weight Path

* i  3.3.3.3/32       192.168.2.1              0    100    300 ?

*>                    192.168.2.1         153856         32768 ?

*>   4.4.4.4/32       0.0.0.0                  0         32768 ?

*>   5.5.5.5/32       192.168.3.2              0    300    500 65003 ?

*>   6.6.6.6/32       192.168.3.2             20    300    500 65003 ?

* i  192.168.1.0/30   192.168.1.5              0    100    300 65001 ?

*>                    192.168.2.1         281856         32768 ?

* i  192.168.1.4/30   192.168.2.1              0    100    300 ?

*>                    192.168.2.1          26112         32768 ?

* i  192.168.2.0/30   192.168.2.1              0    100    300 ?

*>                    0.0.0.0                  0         32768 ?

Network          Next Hop            Metric LocPrf Weight Path

r>   192.168.3.0/30   192.168.3.2              0    300    500 65003 ?

*>   192.168.3.4/30   192.168.3.2              0    300    500 65003 ?

Issues

My groupmate and I encountered an issue where the EIGRP routes weren’t showing, but then we realised that we were stupid and that EIGRP routes naturally don’t show when there are only 2 routers in an EIGRP network. It also explained why the loopback interface routes that we added for troubleshooting was added were showing on the other router but not the one directly connected to it. Later, when the configs were finished, the EIGRP routes did show up on the other routers.

When we were configuring BGP, on the other hand, the link between R4 and R5 was having issues, and it was a weird cabling issue. We then had to re-cable everything just to realise it wasn’t working, so we reloaded the router and it wasn’t working either, so we checked the configs to realise that we didn’t add redistribute commands (if I remember right). After doing that, everything worked again.

Conclusion

Completing this lab helped me get experience on how to set up BGP, helping me with knowledge on how to connect 2 different protocols together. It also honed my troubleshooting skills some more due to the issues that we had in the lab. It also allowed me to gain better reading skills due to how many lines of configurations there are, meaning that missing a single line can cause issues. It also gave me better instinct skills, basically when I feel like something is wrong, it may become true and can allow me to diagnose issues better. Overall, this lab gave me stuff to study, along with stuff to troubleshoot, allowing for me to learn more real-world skills and refine them.